CONTROLALUZ

Firmware Technical Specification

Tabla de contenido

ControlaLuz Firmware	4

Overview & Architecture	4

1. Executive Summary	4

1.1 System Overview	4

1.2 Technology Stack	5

1.3 Key Metrics	5

2. Hardware Architecture	6

2.1 Block Diagram	6

2.2 Pin Configuration	6

2.3 Power Consumption	6

3. Software Architecture	7

3.1 Main Components	7

3.2 State Machine	8

4. Feature Summary	9

4.1 Implemented Features (72 scenarios)	9

4.2 Pending Features (8 scenarios)	9

5. Communication Protocol	9

5.1 MQTT Topics	9

5.2 Message Formats	9

6. Security Architecture	10

6.1 TLS Configuration	10

6.2 Reset Protection	11

7. Memory Management	11

7.1 Flash Layout (4MB)	11

7.2 EEPROM Layout	11

7.3 RAM Usage	12

8. Development Environment	12

8.1 Required Tools	12

8.2 Required Libraries	12

8.3 Compilation	12

9. Deployment Strategy	13

9.1 Initial Flash	13

9.2 OTA Updates	13

10. Testing Strategy	13

WiFi & MQTT Communication	14

1. WiFi Connection Feature	14

1.1 Key Scenarios	14

1.2 Implementation	14

2. MQTT Communication Feature	15

2.1 Key Scenarios	15

2.2 Implementation	16

2.3 Error Handling	19

3. TLS Certificate Management	19

3.1 Certificate Storage	19

3.2 Certificate Rotation	20

4. Network Monitoring	20

4.1 Signal Strength	20

4.2 Bandwidth Monitoring	21

5. Testing Checklist	21

5.1 WiFi Tests	21

5.2 MQTT Tests	21

Sensor Reading & Operating Modes	22

1. Sensor Reading Feature	22

1.1 PZEM-004T Integration	22

1.2 Key Scenarios	23

2. Operating Modes Feature	23

2.1 Mode Definitions	23

2.2 Implementation	23

2.3 Power Consumption by Mode	25

3. Data Validation	25

3.1 Range Validation	25

3.2 Error Recovery	26

4. Testing Checklist	26

4.1 Sensor Tests	26

4.2 Operating Mode Tests	27

OTA Updates & Reset Protection	28

1. OTA Firmware Updates Feature	28

1.1 Key Scenarios	28

1.2 Implementation	28

1.3 Version Management	30

2. Reset Protection Feature	30

2.1 Key Scenarios	30

2.2 Implementation	31

2.3 Security Considerations	33

3. Bootloader & Recovery	33

3.1 Boot Sequence	33

3.2 Rollback on Boot Failure	34

4. Testing Checklist	35

4.1 OTA Tests	35

4.2 Reset Protection Tests	35

Pending Features & Testing	36

1. Pending Features (@pending)	36

1.1 Deep Sleep Mode (8 scenarios)	36

2. LED Indicators Feature	37

2.1 LED States	37

2.2 Implementation	37

3. Complete Testing Strategy	38

3.1 Unit Tests (Arduino Test Framework)	38

3.2 Integration Tests (Serial Monitor)	39

3.3 Load Tests (Multiple Devices)	40

3.4 Power Consumption Tests	40

3.5 Security Tests	41

4. Deployment Guide	41

4.1 Initial Flashing	41

4.2 Configuration (First Boot)	41

4.3 OTA Updates (Production)	42

5. Troubleshooting Guide	43

5.1 Common Issues	43

5.2 Debug Serial Output	43

6. Complete Testing Checklist	44

6.1 Feature Tests	44

6.2 Performance Tests	44

6.3 Security Tests	44

# ControlaLuz Firmware

# Overview & Architecture

Version: 1.0 | Date: October 2025 | Status: Production Ready
Document Series: Firmware Technical Specification (1 of 5)

## 1. Executive Summary

### 1.1 System Overview

ControlaLuz Firmware is an ESP8266-based IoT solution for real-time energy monitoring. The firmware communicates with cloud backend via MQTT over TLS, measures electrical parameters using PZEM-004T sensor, and supports over-the-air (OTA) firmware updates.

Primary Purpose:

- Monitor voltage, current, power, energy consumption

- Transmit readings to cloud via MQTT

- Support multiple operating modes (off/lowpower/interval/realtime)

- Enable remote configuration and OTA updates

- Secure device reset protection

Target Hardware:

- MCU: ESP8266 (NodeMCU, Wemos D1 Mini)

- Sensor: PZEM-004T v3.0 (AC energy meter)

- Communication: WiFi 802.11 b/g/n, MQTT over TLS 1.2

- Power: 5V USB or external power supply

### 1.2 Technology Stack

| Component | Technology | Purpose |
| --- | --- | --- |
| MCU | ESP8266 | WiFi-enabled microcontroller |
| Framework | Arduino Core | Firmware development |
| Protocol | MQTT over TLS 1.2 | Cloud communication |
| Sensor | PZEM-004T | Energy measurement |
| OTA | HTTP/HTTPS | Firmware updates |
| Storage | EEPROM/SPIFFS | Configuration persistence |
| Security | TLS 1.2, SHA-256 | Secure communication |

### 1.3 Key Metrics

| Metric | Value |
| --- | --- |
| Total Features | 7 implemented + 1 pending |
| Scenarios Documented | 72 implemented + 8 pending |
| Firmware Size | ~400 KB |
| RAM Usage | ~20 KB (heap) |
| Power Consumption | ~80 mA (WiFi active) |
| Update Frequency | 2s (realtime) to 5min (lowpower) |

## 2. Hardware Architecture

### 2.1 Block Diagram

┌─────────────────────────────────────────────────┐
│              ESP8266 Module                     │
│  ┌──────────────────────────────────────────┐   │
│  │           Main Controller                │   │
│  │  - WiFi 802.11 b/g/n                     │   │
│  │  - 80 MHz CPU                            │   │
│  │  - 4MB Flash                             │   │
│  │  - 80 KB RAM                             │   │
│  └──────────────────────────────────────────┘   │
│           │                │                    │
│           │ UART           │ GPIO               │
│           ▼                ▼                    │
│  ┌─────────────┐   ┌─────────────┐              │
│  │ PZEM-004T   │   │   LED       │              │
│  │ Sensor      │   │ Indicators  │              │
│  └─────────────┘   └─────────────┘              │
└─────────────────────────────────────────────────┘
           │
           │ WiFi
           ▼
┌─────────────────────────────────────────────────┐
│          AWS IoT Core (MQTT Broker)             │
└─────────────────────────────────────────────────┘

### 2.2 Pin Configuration

// UART for PZEM-004T
#define PZEM_RX_PIN D5  // GPIO14
#define PZEM_TX_PIN D6  // GPIO12

// Status LED
#define LED_PIN D4      // GPIO2 (built-in LED)

// Optional: Factory reset button
#define RESET_BTN_PIN D7  // GPIO13

### 2.3 Power Consumption

| Mode | Power | Description |
| --- | --- | --- |
| Active WiFi | ~80 mA | Transmitting data |
| Idle WiFi | ~15 mA | Connected, not transmitting |
| Deep Sleep | ~20 µA | WiFi off (not implemented) |
| Off Mode | ~5 mA | MCU active, no transmission |

## 3. Software Architecture

### 3.1 Main Components

main.ino
├── Config Management (EEPROM)
│   ├── WiFi credentials
│   ├── MQTT broker URL
│   ├── Device ID (MAC)
│   └── Operating mode
├── WiFi Manager
│   ├── Connection handling
│   ├── Auto-reconnect
│   └── Status monitoring
├── MQTT Client (TLS)
│   ├── Publish readings
│   ├── Subscribe to commands
│   └── Certificate validation
├── PZEM Sensor Driver
│   ├── Read voltage/current
│   ├── Read power/energy
│   └── Error handling
├── Operating Mode Controller
│   ├── Off mode
│   ├── Lowpower mode
│   ├── Interval mode
│   └── Realtime mode
├── OTA Update Manager
│   ├── Download firmware
│   ├── Verify checksum (MD5)
│   └── Flash update
└── Reset Protection
    ├── Admin token validation
    └── Factory reset execution

### 3.2 State Machine

┌─────────────┐
│   STARTUP   │
└──────┬──────┘
       │
       ▼
┌─────────────┐     WiFi Failed      ┌──────────────┐
│ WIFI_INIT   ├─────────────────────►│ CONFIG_MODE  │
└──────┬──────┘                      └──────────────┘
       │ WiFi Connected
       ▼
┌─────────────┐     MQTT Failed      ┌──────────────┐
│ MQTT_INIT   ├─────────────────────►│ RECONNECTING │
└──────┬──────┘                      └──────┬───────┘
       │ MQTT Connected                     │
       ▼                                    │
┌─────────────┐◄────────────────────────────┘
│   RUNNING   │
└──────┬──────┘
       │ Loop
       ▼
┌─────────────┐
│ READ_SENSOR │
└──────┬──────┘
       │
       ▼
┌─────────────┐
│PUBLISH_MQTT │
└──────┬──────┘
       │
       ▼
┌─────────────┐
│    SLEEP    │ (delay based on mode)
└──────┬──────┘
       │
       └──────► (back to RUNNING)

## 4. Feature Summary

### 4.1 Implemented Features (72 scenarios)

| # | Feature | Scenarios | Priority | Description |
| --- | --- | --- | --- | --- |
| 1 | WiFi Connection | 9 | Critical | Network management |
| 2 | MQTT Communication | 10 | Critical | Cloud messaging |
| 3 | Sensor Reading | 12 | Critical | PZEM-004T integration |
| 4 | Operating Modes | 8 | High | Power/frequency control |
| 5 | OTA Updates | 15 | High | Remote firmware updates |
| 6 | Reset Protection | 10 | High | Secure factory reset |
| 7 | LED Indicators | 8 | Medium | Visual status feedback |

### 4.2 Pending Features (8 scenarios)

| # | Feature | Scenarios | Status | Description |
| --- | --- | --- | --- | --- |
| 8 | Deep Sleep Mode | 8 | @pending | Ultra-low power consumption |

## 5. Communication Protocol

### 5.1 MQTT Topics

Device → Cloud:

devices/{deviceId}/readings        # Sensor data
devices/{deviceId}/ota/status      # OTA update progress
devices/{deviceId}/status          # Device status updates

Cloud → Device:

devices/{deviceId}/commands/mode   # Change operating mode
devices/{deviceId}/commands/ota    # Trigger OTA update
devices/{deviceId}/reset/authorize # Authorize factory reset

### 5.2 Message Formats

Reading Payload:

{
  "deviceId": "AA:BB:CC:DD:EE:FF",
  "timestamp": 1698765432000,
  "voltage": 230.5,
  "current": 2.3,
  "power": 530.15,
  "energy": 12.45,
  "frequency": 50.0,
  "powerFactor": 0.95
}

Mode Change Command:

{
  "command": "setMode",
  "operatingMode": "realtime",
  "timestamp": 1698765432
}

OTA Command:

{
  "command": "ota",
  "firmwareUrl": "https://s3.../firmware-1.2.3.bin",
  "version": "1.2.3",
  "md5": "abc123...",
  "deploymentId": "deploy-123"
}

## 6. Security Architecture

### 6.1 TLS Configuration

// AWS IoT Core certificate-based authentication
WiFiClientSecure wifiClient;

// Root CA certificate
const char* root_ca = R"EOF(
-----BEGIN CERTIFICATE-----
MIIDQTCCAimgAwIBAgITBmyfz5m/jAo54vB4ikPmljZbyjANBgkqhkiG9w0BAQsF
...
-----END CERTIFICATE-----
)EOF";

// Device certificate
const char* certificate = R"EOF(
-----BEGIN CERTIFICATE-----
MIIDWjCCAkKgAwIBAgIVAKBjk...
-----END CERTIFICATE-----
)EOF";

// Device private key
const char* private_key = R"EOF(
-----BEGIN RSA PRIVATE KEY-----
MIIEowIBAAKCAQEAvZ...
-----END RSA PRIVATE KEY-----
)EOF";

// Setup TLS
wifiClient.setCACert(root_ca);
wifiClient.setCertificate(certificate);
wifiClient.setPrivateKey(private_key);

### 6.2 Reset Protection

// Factory reset requires admin authorization
bool validateResetToken(const char* token, const char* adminId) {
  // Generate expected token using SHA-256
  char expectedToken[65];
  sha256(String(deviceId) + adminId + resetSecret, expectedToken);
  
  return strcmp(token, expectedToken) == 0;
}

// Only allow reset with valid token
if (validateResetToken(receivedToken, adminId)) {
  factoryReset();
} else {
  Serial.println("Reset denied: Invalid token");
}

## 7. Memory Management

### 7.1 Flash Layout (4MB)

0x000000 - 0x010000  (64 KB)   Bootloader
0x010000 - 0x070000  (384 KB)  Sketch (firmware)
0x070000 - 0x0D0000  (384 KB)  OTA partition
0x0D0000 - 0x0E0000  (64 KB)   SPIFFS (filesystem)
0x0E0000 - 0x0F0000  (64 KB)   EEPROM emulation
0x0F0000 - 0x100000  (64 KB)   WiFi config

### 7.2 EEPROM Layout

// Address map
#define EEPROM_SIZE 512

#define ADDR_WIFI_SSID       0    // 32 bytes
#define ADDR_WIFI_PASSWORD   32   // 64 bytes
#define ADDR_MQTT_BROKER     96   // 128 bytes
#define ADDR_DEVICE_ID       224  // 18 bytes (MAC)
#define ADDR_OPERATING_MODE  242  // 1 byte
#define ADDR_CHECKSUM        256  // 32 bytes (SHA-256)

### 7.3 RAM Usage

Stack:           ~4 KB
Heap (dynamic):  ~20 KB
  - WiFi buffers:  8 KB
  - MQTT buffers:  4 KB
  - JSON buffers:  2 KB
  - Variables:     6 KB
Total available: ~80 KB

## 8. Development Environment

### 8.1 Required Tools

| Tool | Version | Purpose |
| --- | --- | --- |
| Arduino IDE | 2.x | Firmware development |
| ESP8266 Core | 3.x | Board support |
| PlatformIO | Optional | Advanced IDE |
| esptool.py | Latest | Flash utility |

### 8.2 Required Libraries

// Platform
#include <ESP8266WiFi.h>
#include <ESP8266HTTPClient.h>
#include <ESP8266httpUpdate.h>

// MQTT
#include <PubSubClient.h>

// Sensor
#include <PZEM004Tv30.h>

// JSON
#include <ArduinoJson.h>

// Security
#include <Hash.h>  // SHA-256

// Storage
#include <EEPROM.h>
#include <FS.h>

### 8.3 Compilation

# Arduino IDE
arduino-cli compile --fqbn esp8266:esp8266:nodemcuv2 firmware.ino

# PlatformIO
pio run

# Flash to device
esptool.py --port /dev/ttyUSB0 write_flash 0x0 firmware.bin

## 9. Deployment Strategy

### 9.1 Initial Flash

- Connect ESP8266 via USB

- Enter bootloader mode (hold FLASH button)

- Flash firmware using esptool.py

- Embed certificates in code

- Configure WiFi credentials

- Test connectivity

### 9.2 OTA Updates

- Admin uploads firmware to S3 (backend)

- Backend sends MQTT command to device

- Device downloads firmware over HTTPS

- Device verifies MD5 checksum

- Device flashes new firmware

- Device reboots with new version

- Device reports status to backend

## 10. Testing Strategy

| Type | Tool | Purpose |
| --- | --- | --- |
| Unit Tests | Arduino Test Framework | Component testing |
| Integration | Serial Monitor | End-to-end flow |
| Load Tests | Multiple devices | MQTT throughput |
| Power Tests | Multimeter | Current consumption |
| Security | SSL Labs | TLS validation |

# WiFi & MQTT Communication

## 1. WiFi Connection Feature

Feature File: wifi-connection.feature
Scenarios: 9
Priority: Critical

### 1.1 Key Scenarios

| # | Scenario | Expected Result |
| --- | --- | --- |
| 1 | Connect to WiFi | SSID/password from EEPROM |
| 2 | Failed connection | Retry 3 times, fallback to AP mode |
| 3 | Lost connection | Auto-reconnect |
| 4 | Weak signal | Monitor RSSI, alert if < -80 dBm |
| 5 | Multiple networks | Connect to strongest |
| 6 | Static IP | Configure static IP (optional) |
| 7 | DNS resolution | Resolve MQTT broker hostname |
| 8 | Connection timeout | 30 seconds max |
| 9 | Network scan | List available networks |

### 1.2 Implementation

// WiFi credentials (stored in EEPROM)
char wifi_ssid[32];
char wifi_password[64];

void setup_wifi() {
  Serial.print("Connecting to WiFi: ");
  Serial.println(wifi_ssid);
  
  WiFi.mode(WIFI_STA);
  WiFi.begin(wifi_ssid, wifi_password);
  
  int attempts = 0;
  while (WiFi.status() != WL_CONNECTED && attempts < 30) {
    delay(1000);
    Serial.print(".");
    attempts++;
    
    // Blink LED while connecting
    digitalWrite(LED_PIN, !digitalRead(LED_PIN));
  }
  
  if (WiFi.status() == WL_CONNECTED) {
    Serial.println("\nWiFi connected");
    Serial.print("IP: ");
    Serial.println(WiFi.localIP());
    Serial.print("RSSI: ");
    Serial.println(WiFi.RSSI());
    
    // LED solid on
    digitalWrite(LED_PIN, HIGH);
  } else {
    Serial.println("\nWiFi connection failed");
    // Enter AP mode for configuration
    startAPMode();
  }
}

// Auto-reconnect if connection lost
void check_wifi_connection() {
  if (WiFi.status() != WL_CONNECTED) {
    Serial.println("WiFi disconnected, reconnecting...");
    setup_wifi();
  }
}

// Access Point mode for initial configuration
void startAPMode() {
  WiFi.mode(WIFI_AP);
  WiFi.softAP("ControlaLuz-Setup", "12345678");
  
  Serial.println("AP Mode started");
  Serial.print("AP IP: ");
  Serial.println(WiFi.softAPIP());
  
  // Start web server for configuration (not shown)
  startWebServer();
}

## 2. MQTT Communication Feature

Feature File: mqtt-communication.feature
Scenarios: 10
Priority: Critical

### 2.1 Key Scenarios

| # | Scenario | Expected Result |
| --- | --- | --- |
| 1 | Connect to broker | TLS with certificates |
| 2 | Publish readings | JSON payload to topic |
| 3 | Subscribe to commands | Receive mode changes, OTA |
| 4 | QoS 1 delivery | At-least-once guarantee |
| 5 | Connection lost | Auto-reconnect with backoff |
| 6 | Large payloads | Handle up to 2 KB |
| 7 | Keep-alive | 60 seconds ping |
| 8 | Last Will | Notify offline status |
| 9 | Retained messages | Persist status |
| 10 | Certificate validation | Verify broker identity |

### 2.2 Implementation

#include <PubSubClient.h>

WiFiClientSecure wifiClient;
PubSubClient mqttClient(wifiClient);

const char* mqtt_broker = "xxxx.iot.us-east-1.amazonaws.com";
const int mqtt_port = 8883;

// Topics
char topic_readings[64];
char topic_commands_mode[64];
char topic_commands_ota[64];
char topic_reset_authorize[64];

void setup_mqtt() {
  // Set TLS certificates
  wifiClient.setCACert(root_ca);
  wifiClient.setCertificate(certificate);
  wifiClient.setPrivateKey(private_key);
  
  // Set MQTT broker
  mqttClient.setServer(mqtt_broker, mqtt_port);
  mqttClient.setCallback(mqtt_callback);
  mqttClient.setBufferSize(2048);  // Increase for large messages
  
  // Build topics
  String deviceId = WiFi.macAddress();
  snprintf(topic_readings, 64, "devices/%s/readings", deviceId.c_str());
  snprintf(topic_commands_mode, 64, "devices/%s/commands/mode", deviceId.c_str());
  snprintf(topic_commands_ota, 64, "devices/%s/commands/ota", deviceId.c_str());
  snprintf(topic_reset_authorize, 64, "devices/%s/reset/authorize", deviceId.c_str());
  
  connect_mqtt();
}

void connect_mqtt() {
  while (!mqttClient.connected()) {
    Serial.print("Connecting to MQTT...");
    
    String clientId = "ControlaLuz-" + WiFi.macAddress();
    
    // Last Will Testament
    char topic_status[64];
    snprintf(topic_status, 64, "devices/%s/status", WiFi.macAddress().c_str());
    
    if (mqttClient.connect(clientId.c_str(), 
                          topic_status, 
                          1,  // QoS
                          true,  // Retain
                          "{\"status\":\"offline\"}")) {
      Serial.println("connected");
      
      // Subscribe to command topics
      mqttClient.subscribe(topic_commands_mode, 1);
      mqttClient.subscribe(topic_commands_ota, 1);
      mqttClient.subscribe(topic_reset_authorize, 1);
      
      // Publish online status
      mqttClient.publish(topic_status, "{\"status\":\"online\"}", true);
    } else {
      Serial.print("failed, rc=");
      Serial.println(mqttClient.state());
      delay(5000);
    }
  }
}

// Publish sensor reading
void publish_reading(float voltage, float current, float power, float energy) {
  StaticJsonDocument<256> doc;
  
  doc["deviceId"] = WiFi.macAddress();
  doc["timestamp"] = millis();  // Replace with NTP timestamp
  doc["voltage"] = voltage;
  doc["current"] = current;
  doc["power"] = power;
  doc["energy"] = energy;
  doc["frequency"] = 50.0;  // Placeholder
  doc["powerFactor"] = 0.95;  // Placeholder
  
  char payload[256];
  serializeJson(doc, payload);
  
  mqttClient.publish(topic_readings, payload, false);
  Serial.println("Reading published");
}

// MQTT callback for incoming messages
void mqtt_callback(char* topic, byte* payload, unsigned int length) {
  Serial.print("Message received on topic: ");
  Serial.println(topic);
  
  // Parse JSON
  StaticJsonDocument<512> doc;
  DeserializationError error = deserializeJson(doc, payload, length);
  
  if (error) {
    Serial.println("JSON parse error");
    return;
  }
  
  const char* command = doc["command"];
  
  // Handle mode change
  if (strcmp(topic, topic_commands_mode) == 0) {
    const char* newMode = doc["operatingMode"];
    change_operating_mode(newMode);
  }
  
  // Handle OTA
  if (strcmp(topic, topic_commands_ota) == 0) {
    const char* firmwareUrl = doc["firmwareUrl"];
    const char* md5 = doc["md5"];
    start_ota_update(firmwareUrl, md5);
  }
  
  // Handle reset authorization
  if (strcmp(topic, topic_reset_authorize) == 0) {
    const char* token = doc["token"];
    const char* adminId = doc["admin_id"];
    if (validate_reset_token(token, adminId)) {
      factory_reset();
    }
  }
}

// Main loop: check MQTT connection
void loop() {
  if (!mqttClient.connected()) {
    connect_mqtt();
  }
  mqttClient.loop();
  
  // ... rest of loop
}

### 2.3 Error Handling

// MQTT connection states
#define MQTT_CONNECTION_TIMEOUT -4
#define MQTT_CONNECTION_LOST -3
#define MQTT_CONNECT_FAILED -2
#define MQTT_DISCONNECTED -1
#define MQTT_CONNECTED 0
#define MQTT_CONNECT_BAD_PROTOCOL 1
#define MQTT_CONNECT_BAD_CLIENT_ID 2
#define MQTT_CONNECT_UNAVAILABLE 3
#define MQTT_CONNECT_BAD_CREDENTIALS 4
#define MQTT_CONNECT_UNAUTHORIZED 5

void handle_mqtt_error(int state) {
  switch (state) {
    case MQTT_CONNECTION_TIMEOUT:
      Serial.println("MQTT timeout");
      break;
    case MQTT_CONNECT_BAD_CREDENTIALS:
      Serial.println("MQTT bad credentials - check certificates");
      break;
    case MQTT_CONNECT_UNAUTHORIZED:
      Serial.println("MQTT unauthorized - check IAM policy");
      break;
    default:
      Serial.print("MQTT error: ");
      Serial.println(state);
  }
}

## 3. TLS Certificate Management

### 3.1 Certificate Storage

// Option 1: Hardcoded in code (development)
const char* root_ca = R"EOF(
-----BEGIN CERTIFICATE-----
MIIDQTCCAimgAwIBAgITBmyfz5m/jAo54vB4ikPmljZbyjANBgkqhkiG9w0BAQsF
ADA5MQswCQYDVQQGEwJVUzEPMA0GA1UEChMGQW1hem9uMRkwFwYDVQQDExBBbWF6
b24gUm9vdCBDQSAxMB4XDTE1MDUyNjAwMDAwMFoXDTM4MDExNzAwMDAwMFowOTEL
...
-----END CERTIFICATE-----
)EOF";

// Option 2: Load from SPIFFS (production)
String loadCertificate(const char* filename) {
  File file = SPIFFS.open(filename, "r");
  if (!file) {
    Serial.println("Failed to open certificate file");
    return "";
  }
  String cert = file.readString();
  file.close();
  return cert;
}

### 3.2 Certificate Rotation

// Check certificate expiry
bool isCertificateValid() {
  // Parse certificate and check expiry date
  // This is simplified - actual implementation would use mbedtls
  return true;  // Placeholder
}

// Update certificate via OTA
void updateCertificate(const char* newCert) {
  File file = SPIFFS.open("/cert.pem", "w");
  file.print(newCert);
  file.close();
  
  // Reconnect MQTT with new certificate
  mqttClient.disconnect();
  setup_mqtt();
}

## 4. Network Monitoring

### 4.1 Signal Strength

void monitor_wifi_signal() {
  int rssi = WiFi.RSSI();
  
  if (rssi > -50) {
    Serial.println("Signal: Excellent");
  } else if (rssi > -60) {
    Serial.println("Signal: Good");
  } else if (rssi > -70) {
    Serial.println("Signal: Fair");
  } else if (rssi > -80) {
    Serial.println("Signal: Weak");
  } else {
    Serial.println("Signal: Very Weak - connection issues likely");
  }
}

### 4.2 Bandwidth Monitoring

unsigned long bytes_sent = 0;
unsigned long bytes_received = 0;

void track_bandwidth(unsigned int sent, unsigned int received) {
  bytes_sent += sent;
  bytes_received += received;
  
  // Log every 1000 messages
  if (bytes_sent % 100000 == 0) {
    Serial.print("Bandwidth - Sent: ");
    Serial.print(bytes_sent / 1024);
    Serial.print(" KB, Received: ");
    Serial.print(bytes_received / 1024);
    Serial.println(" KB");
  }
}

## 5. Testing Checklist

### 5.1 WiFi Tests

- Connect to WPA2 network

- Handle wrong password

- Auto-reconnect after disconnect

- Handle weak signal

- Static IP configuration

- AP mode for setup

- Multiple network handling

### 5.2 MQTT Tests

- Connect to AWS IoT Core

- Publish readings (QoS 1)

- Subscribe to commands

- Receive mode change command

- Handle connection loss

- Validate TLS certificates

- Last Will Testament

- Large payload handling

# Sensor Reading & Operating Modes

## 1. Sensor Reading Feature

Feature File: sensor-reading.feature
Scenarios: 12
Priority: Critical

### 1.1 PZEM-004T Integration

#include <PZEM004Tv30.h>

PZEM004Tv30 pzem(PZEM_RX_PIN, PZEM_TX_PIN);

void setup() {
  Serial.begin(115200);
  // PZEM uses SoftwareSerial at 9600 baud
}

struct SensorReading {
  float voltage;      // V
  float current;      // A
  float power;        // W
  float energy;       // kWh
  float frequency;    // Hz
  float powerFactor;  // 0.0-1.0
  bool valid;
};

SensorReading read_sensor() {
  SensorReading reading;
  
  reading.voltage = pzem.voltage();
  reading.current = pzem.current();
  reading.power = pzem.power();
  reading.energy = pzem.energy();
  reading.frequency = pzem.frequency();
  reading.powerFactor = pzem.pf();
  
  // Check if reading is valid (NaN check)
  reading.valid = !isnan(reading.voltage) && 
                  !isnan(reading.current) && 
                  !isnan(reading.power);
  
  if (!reading.valid) {
    Serial.println("PZEM read error");
  }
  
  return reading;
}

### 1.2 Key Scenarios

| # | Scenario | Expected Result |
| --- | --- | --- |
| 1 | Read voltage | 220-240V AC |
| 2 | Read current | 0-100A |
| 3 | Read power | 0-22kW |
| 4 | Read energy | Accumulated kWh |
| 5 | Read frequency | 50Hz or 60Hz |
| 6 | Read power factor | 0.0-1.0 |
| 7 | Sensor timeout | Return error after 3s |
| 8 | Invalid reading | Retry 3 times |
| 9 | Reset energy counter | Admin command |
| 10 | Calibration | Not supported by PZEM |
| 11 | Multiple sensors | Separate UART (future) |
| 12 | Error logging | Store in SPIFFS |

## 2. Operating Modes Feature

Feature File: operating-modes.feature
Scenarios: 8
Priority: High

### 2.1 Mode Definitions

| Mode | Reading Interval | Use Case |
| --- | --- | --- |
| off | No readings | Device maintenance |
| lowpower | 5 minutes | Battery-powered (future) |
| interval | 15 seconds | Normal monitoring |
| realtime | 2 seconds | Debugging, real-time graphs |

### 2.2 Implementation

enum OperatingMode {
  MODE_OFF,
  MODE_LOWPOWER,
  MODE_INTERVAL,
  MODE_REALTIME
};

OperatingMode current_mode = MODE_INTERVAL;
unsigned long last_reading_time = 0;

void change_operating_mode(const char* mode_str) {
  OperatingMode new_mode;
  
  if (strcmp(mode_str, "off") == 0) {
    new_mode = MODE_OFF;
  } else if (strcmp(mode_str, "lowpower") == 0) {
    new_mode = MODE_LOWPOWER;
  } else if (strcmp(mode_str, "interval") == 0) {
    new_mode = MODE_INTERVAL;
  } else if (strcmp(mode_str, "realtime") == 0) {
    new_mode = MODE_REALTIME;
  } else {
    Serial.println("Invalid mode");
    return;
  }
  
  current_mode = new_mode;
  
  // Save to EEPROM
  EEPROM.write(ADDR_OPERATING_MODE, (byte)new_mode);
  EEPROM.commit();
  
  Serial.print("Mode changed to: ");
  Serial.println(mode_str);
  
  // Publish acknowledgment
  publish_mode_ack(mode_str);
}

unsigned long get_reading_interval() {
  switch (current_mode) {
    case MODE_OFF:
      return 0;  // No readings
    case MODE_LOWPOWER:
      return 300000;  // 5 minutes
    case MODE_INTERVAL:
      return 15000;   // 15 seconds
    case MODE_REALTIME:
      return 2000;    // 2 seconds
    default:
      return 15000;
  }
}

void loop() {
  // Check WiFi
  check_wifi_connection();
  
  // Check MQTT
  if (!mqttClient.connected()) {
    connect_mqtt();
  }
  mqttClient.loop();
  
  // Read and publish based on mode
  unsigned long interval = get_reading_interval();
  
  if (interval > 0 && millis() - last_reading_time >= interval) {
    SensorReading reading = read_sensor();
    
    if (reading.valid) {
      publish_reading(reading.voltage, reading.current, 
                     reading.power, reading.energy);
    }
    
    last_reading_time = millis();
  }
  
  // LED blink to show activity
  if (current_mode == MODE_REALTIME) {
    digitalWrite(LED_PIN, !digitalRead(LED_PIN));
  }
}

### 2.3 Power Consumption by Mode

| Mode | Current Draw | Daily Energy |
| --- | --- | --- |
| Off | ~5 mA | ~1 Wh |
| Lowpower | ~15 mA avg | ~3.6 Wh |
| Interval | ~30 mA avg | ~7.2 Wh |
| Realtime | ~80 mA avg | ~19.2 Wh |

## 3. Data Validation

### 3.1 Range Validation

bool validate_reading(SensorReading reading) {
  // Voltage: 180-260V (allow for fluctuations)
  if (reading.voltage < 180 || reading.voltage > 260) {
    Serial.println("Voltage out of range");
    return false;
  }
  
  // Current: 0-100A
  if (reading.current < 0 || reading.current > 100) {
    Serial.println("Current out of range");
    return false;
  }
  
  // Power: 0-22000W
  if (reading.power < 0 || reading.power > 22000) {
    Serial.println("Power out of range");
    return false;
  }
  
  // Frequency: 45-65Hz
  if (reading.frequency < 45 || reading.frequency > 65) {
    Serial.println("Frequency out of range");
    return false;
  }
  
  return true;
}

### 3.2 Error Recovery

int sensor_error_count = 0;
const int MAX_SENSOR_ERRORS = 10;

void handle_sensor_error() {
  sensor_error_count++;
  
  if (sensor_error_count >= MAX_SENSOR_ERRORS) {
    Serial.println("Too many sensor errors, restarting...");
    
    // Publish error status
    publish_device_error("Sensor failure");
    
    // Restart ESP8266
    ESP.restart();
  }
}

void reset_error_count() {
  sensor_error_count = 0;
}

## 4. Testing Checklist

### 4.1 Sensor Tests

- Read voltage correctly

- Read current correctly

- Read power correctly

- Read accumulated energy

- Handle sensor disconnect

- Validate reading ranges

- Retry on error

- Reset energy counter

### 4.2 Operating Mode Tests

- Switch to off mode (stop readings)

- Switch to lowpower (5 min interval)

- Switch to interval (15 sec)

- Switch to realtime (2 sec)

- Persist mode to EEPROM

- Acknowledge mode change via MQTT

- Invalid mode rejection

- Power consumption verification

# OTA Updates & Reset Protection

## 1. OTA Firmware Updates Feature

Feature File: ota-updates.feature
Scenarios: 15
Priority: High

### 1.1 Key Scenarios

| # | Scenario | Expected Result |
| --- | --- | --- |
| 1 | Receive OTA command | Download firmware from S3 |
| 2 | Verify MD5 checksum | Match before flashing |
| 3 | Flash firmware | Write to OTA partition |
| 4 | Reboot with new version | Verify boot success |
| 5 | Report status | Publish progress via MQTT |
| 6 | Handle download failure | Retry 3 times, rollback |
| 7 | Handle flash failure | Keep old firmware |
| 8 | Low battery protection | Skip OTA if power low (future) |
| 9 | Resume interrupted download | Not supported |
| 10 | Version check | Don’t downgrade |

### 1.2 Implementation

#include <ESP8266httpUpdate.h>

void start_ota_update(const char* firmwareUrl, const char* md5) {
  Serial.println("Starting OTA update...");
  
  // Publish status: downloading
  publish_ota_status("downloading", "0");
  
  // Configure HTTP client
  WiFiClientSecure client;
  client.setInsecure();  // For S3 presigned URLs
  
  // Set MD5 for verification
  ESPhttpUpdate.setLedPin(LED_PIN, LOW);
  
  // Perform update
  t_httpUpdate_return ret = ESPhttpUpdate.update(client, firmwareUrl, "", md5);
  
  switch (ret) {
    case HTTP_UPDATE_FAILED:
      Serial.printf("Update failed: %s\n", ESPhttpUpdate.getLastErrorString().c_str());
      publish_ota_status("failed", ESPhttpUpdate.getLastErrorString().c_str());
      break;
      
    case HTTP_UPDATE_NO_UPDATES:
      Serial.println("No update available");
      publish_ota_status("no_updates", "");
      break;
      
    case HTTP_UPDATE_OK:
      Serial.println("Update successful, rebooting...");
      publish_ota_status("success", "");
      delay(1000);
      ESP.restart();
      break;
  }
}

void publish_ota_status(const char* status, const char* message) {
  StaticJsonDocument<256> doc;
  doc["status"] = status;
  doc["message"] = message;
  doc["timestamp"] = millis();
  doc["version"] = FIRMWARE_VERSION;
  
  char payload[256];
  serializeJson(doc, payload);
  
  char topic[64];
  snprintf(topic, 64, "devices/%s/ota/status", WiFi.macAddress().c_str());
  
  mqttClient.publish(topic, payload);
}

// Optional: Monitor download progress
void update_progress(int progress, int total) {
  int percentage = (progress * 100) / total;
  
  static int last_percentage = 0;
  if (percentage - last_percentage >= 10) {
    Serial.printf("Download progress: %d%%\n", percentage);
    
    char progress_str[8];
    snprintf(progress_str, 8, "%d", percentage);
    publish_ota_status("downloading", progress_str);
    
    last_percentage = percentage;
  }
}

### 1.3 Version Management

#define FIRMWARE_VERSION "1.2.3"

void check_version_on_boot() {
  Serial.print("Firmware version: ");
  Serial.println(FIRMWARE_VERSION);
  
  // Read previous version from EEPROM
  char prev_version[16];
  EEPROM.get(ADDR_FIRMWARE_VERSION, prev_version);
  
  if (strcmp(prev_version, FIRMWARE_VERSION) != 0) {
    Serial.println("New firmware detected");
    
    // Save new version
    EEPROM.put(ADDR_FIRMWARE_VERSION, FIRMWARE_VERSION);
    EEPROM.commit();
    
    // Publish version update
    publish_version_update(prev_version, FIRMWARE_VERSION);
  }
}

## 2. Reset Protection Feature

Feature File: reset-protection.feature
Scenarios: 10
Priority: High

### 2.1 Key Scenarios

| # | Scenario | Expected Result |
| --- | --- | --- |
| 1 | Admin initiates reset | Publish MQTT command with token |
| 2 | Device validates token | SHA-256 hash verification |
| 3 | Valid token | Execute factory reset |
| 4 | Invalid token | Reject reset |
| 5 | Token expires | 5 minute timeout |
| 6 | Physical button reset | Require 10-second press |
| 7 | Reset clears WiFi | Return to AP mode |
| 8 | Reset clears MQTT config | Require re-pairing |
| 9 | Reset preserves device ID | MAC address stays |
| 10 | Log reset event | Store in SPIFFS |

### 2.2 Implementation

#include <Hash.h>

const char* RESET_SECRET = "CHANGE_ME_IN_PRODUCTION";

void mqtt_callback(char* topic, byte* payload, unsigned int length) {
  // ... (previous code)
  
  // Handle reset authorization
  if (strcmp(topic, topic_reset_authorize) == 0) {
    StaticJsonDocument<256> doc;
    deserializeJson(doc, payload, length);
    
    const char* token = doc["token"];
    const char* admin_id = doc["admin_id"];
    unsigned long timestamp = doc["timestamp"];
    const char* reason = doc["reason"];
    
    // Check token expiry (5 minutes)
    unsigned long current_time = millis() / 1000;
    if (abs((long)(current_time - timestamp)) > 300) {
      Serial.println("Reset token expired");
      publish_reset_status("denied", "Token expired");
      return;
    }
    
    // Validate token
    if (validate_reset_token(token, admin_id, timestamp)) {
      Serial.print("Factory reset authorized by: ");
      Serial.println(admin_id);
      Serial.print("Reason: ");
      Serial.println(reason);
      
      publish_reset_status("accepted", "Resetting...");
      delay(1000);
      
      factory_reset();
    } else {
      Serial.println("Reset denied: Invalid token");
      publish_reset_status("denied", "Invalid token");
    }
  }
}

bool validate_reset_token(const char* token, const char* admin_id, unsigned long timestamp) {
  // Generate expected token
  String data = String(WiFi.macAddress()) + admin_id + timestamp + RESET_SECRET;
  String expected_token = sha256(data);
  
  return (strcmp(token, expected_token.c_str()) == 0);
}

void factory_reset() {
  Serial.println("FACTORY RESET - Clearing all settings");
  
  // Clear EEPROM
  for (int i = 0; i < EEPROM_SIZE; i++) {
    EEPROM.write(i, 0);
  }
  EEPROM.commit();
  
  // Clear SPIFFS
  SPIFFS.format();
  
  // Clear WiFi credentials
  WiFi.disconnect(true);
  
  Serial.println("Reset complete, rebooting...");
  delay(1000);
  ESP.restart();
}

void publish_reset_status(const char* status, const char* message) {
  StaticJsonDocument<128> doc;
  doc["status"] = status;
  doc["message"] = message;
  
  char payload[128];
  serializeJson(doc, payload);
  
  char topic[64];
  snprintf(topic, 64, "devices/%s/reset/status", WiFi.macAddress().c_str());
  
  mqttClient.publish(topic, payload);
}

// Physical button reset (10-second press)
void check_physical_reset() {
  static unsigned long button_press_start = 0;
  static bool button_was_pressed = false;
  
  bool button_pressed = (digitalRead(RESET_BTN_PIN) == LOW);
  
  if (button_pressed && !button_was_pressed) {
    button_press_start = millis();
    Serial.println("Reset button pressed");
  }
  
  if (button_pressed && millis() - button_press_start >= 10000) {
    Serial.println("Physical reset triggered");
    factory_reset();
  }
  
  if (!button_pressed) {
    button_press_start = 0;
  }
  
  button_was_pressed = button_pressed;
}

### 2.3 Security Considerations

Token Generation (Backend):

// Backend generates token
const crypto = require('crypto');

function generateResetToken(deviceId, adminId, timestamp, secret) {
  const data = `${deviceId}${adminId}${timestamp}${secret}`;
  return crypto.createHash('sha256').update(data).digest('hex');
}

// Send to device via MQTT
const token = generateResetToken(deviceId, adminId, Math.floor(Date.now()/1000), SECRET);
await iotData.publish({
  topic: `devices/${deviceId}/reset/authorize`,
  payload: JSON.stringify({ token, admin_id: adminId, timestamp, reason })
});

## 3. Bootloader & Recovery

### 3.1 Boot Sequence

void setup() {
  Serial.begin(115200);
  Serial.println("\n\n=== ControlaLuz Boot ===");
  
  // Check reset reason
  Serial.print("Reset reason: ");
  Serial.println(ESP.getResetReason());
  
  // Check if OTA update succeeded
  if (ESP.getResetReason() == "Software/System restart") {
    Serial.println("OTA update successful");
  }
  
  // Initialize EEPROM
  EEPROM.begin(EEPROM_SIZE);
  
  // Load configuration
  load_config();
  
  // Check firmware version
  check_version_on_boot();
  
  // Setup WiFi
  setup_wifi();
  
  // Setup MQTT
  setup_mqtt();
  
  Serial.println("Boot complete");
}

### 3.2 Rollback on Boot Failure

// ESP8266 automatically rolls back if new firmware doesn't boot
// No additional code needed - handled by bootloader

// Optional: Health check on first boot
bool first_boot_after_ota = false;

void setup() {
  // ... (boot sequence)
  
  if (first_boot_after_ota) {
    delay(5000);  // Give time for stability check
    
    if (mqtt_connected && sensor_working) {
      Serial.println("OTA successful, firmware stable");
      publish_ota_status("verified", "Firmware working correctly");
      first_boot_after_ota = false;
    } else {
      Serial.println("OTA failed, firmware unstable - rollback needed");
      // Bootloader will handle rollback on next restart
      ESP.restart();
    }
  }
}

## 4. Testing Checklist

### 4.1 OTA Tests

- Successful OTA update

- MD5 checksum verification

- Download failure handling

- Flash failure handling

- Version check (no downgrade)

- Progress reporting

- Reboot after update

- Rollback on failure

### 4.2 Reset Protection Tests

- Admin reset with valid token

- Reject invalid token

- Token expiry (5 minutes)

- Physical button reset (10 sec)

- Clear WiFi credentials

- Clear MQTT config

- Preserve device ID (MAC)

- Log reset event

# Pending Features & Testing

## 1. Pending Features (@pending)

### 1.1 Deep Sleep Mode (8 scenarios)

Feature File: deep-sleep.feature
Status: @pending (ultra-low power optimization)
Priority: Medium

Key Scenarios: 1. Enter deep sleep between readings 2. Wake up via timer (15 min) 3. Power consumption < 20 µA 4. Quick wake (< 1 second) 5. Preserve WiFi credentials 6. Resume MQTT connection 7. Battery monitoring (future) 8. Solar panel integration (future)

Implementation Sketch:

// @pending - Not yet implemented
void enter_deep_sleep(unsigned long sleep_seconds) {
  Serial.println("Entering deep sleep");
  
  // Disconnect MQTT gracefully
  mqttClient.disconnect();
  
  // Disconnect WiFi
  WiFi.disconnect();
  
  // Enter deep sleep (GPIO16 must be connected to RST for wake)
  ESP.deepSleep(sleep_seconds * 1000000);
  // Device will reset after wake
}

void setup() {
  // After deep sleep wake, this runs again
  // Quick reconnect optimization needed
  
  if (ESP.getResetReason() == "Deep-Sleep Wake") {
    Serial.println("Woke from deep sleep");
    quick_reconnect();
  }
}

Hardware Requirement:

- Connect GPIO16 (D0) to RST pin for wake-up

- External wake sources not supported on ESP8266

## 2. LED Indicators Feature

Feature File: led-indicators.feature
Scenarios: 8
Priority: Medium
Status: Implemented

### 2.1 LED States

| State | LED Pattern | Meaning |
| --- | --- | --- |
| Booting | Fast blink (200ms) | Device starting |
| WiFi connecting | Slow blink (1s) | Connecting to network |
| WiFi connected | Solid ON | Connected, waiting MQTT |
| MQTT connected | Solid ON | Fully operational |
| Reading | Quick flash | Sensor reading |
| OTA update | Fast blink | Downloading firmware |
| Error | Fast blink 3x, pause | Error condition |
| AP mode | Alternating blink | Setup mode |

### 2.2 Implementation

enum LEDState {
  LED_BOOT,
  LED_WIFI_CONNECTING,
  LED_WIFI_CONNECTED,
  LED_MQTT_CONNECTED,
  LED_READING,
  LED_OTA,
  LED_ERROR,
  LED_AP_MODE
};

LEDState led_state = LED_BOOT;
unsigned long last_led_toggle = 0;
bool led_on = false;

void update_led() {
  unsigned long now = millis();
  unsigned long interval = 0;
  
  switch (led_state) {
    case LED_BOOT:
      interval = 200;
      break;
    case LED_WIFI_CONNECTING:
      interval = 1000;
      break;
    case LED_WIFI_CONNECTED:
    case LED_MQTT_CONNECTED:
      digitalWrite(LED_PIN, HIGH);
      return;
    case LED_READING:
      // Single flash
      digitalWrite(LED_PIN, HIGH);
      delay(50);
      digitalWrite(LED_PIN, LOW);
      return;
    case LED_OTA:
      interval = 100;
      break;
    case LED_ERROR:
      // 3 fast blinks, then pause
      for (int i = 0; i < 3; i++) {
        digitalWrite(LED_PIN, HIGH);
        delay(100);
        digitalWrite(LED_PIN, LOW);
        delay(100);
      }
      delay(1000);
      return;
    case LED_AP_MODE:
      interval = 500;
      break;
  }
  
  if (now - last_led_toggle >= interval) {
    led_on = !led_on;
    digitalWrite(LED_PIN, led_on ? HIGH : LOW);
    last_led_toggle = now;
  }
}

void loop() {
  update_led();
  // ... rest of loop
}

## 3. Complete Testing Strategy

### 3.1 Unit Tests (Arduino Test Framework)

// test/test_sensor.cpp
#include <unity.h>

void test_sensor_reading_valid() {
  SensorReading reading = read_sensor();
  TEST_ASSERT_TRUE(reading.valid);
  TEST_ASSERT_GREATER_THAN(180.0, reading.voltage);
  TEST_ASSERT_LESS_THAN(260.0, reading.voltage);
}

void test_sensor_range_validation() {
  SensorReading reading;
  reading.voltage = 300.0;  // Out of range
  TEST_ASSERT_FALSE(validate_reading(reading));
}

void setup() {
  UNITY_BEGIN();
  RUN_TEST(test_sensor_reading_valid);
  RUN_TEST(test_sensor_range_validation);
  UNITY_END();
}

### 3.2 Integration Tests (Serial Monitor)

Test Script:

void run_integration_tests() {
  Serial.println("\n=== INTEGRATION TESTS ===");
  
  // Test 1: WiFi connection
  Serial.print("Test WiFi: ");
  setup_wifi();
  if (WiFi.status() == WL_CONNECTED) {
    Serial.println("PASS");
  } else {
    Serial.println("FAIL");
  }
  
  // Test 2: MQTT connection
  Serial.print("Test MQTT: ");
  connect_mqtt();
  if (mqttClient.connected()) {
    Serial.println("PASS");
  } else {
    Serial.println("FAIL");
  }
  
  // Test 3: Sensor reading
  Serial.print("Test Sensor: ");
  SensorReading reading = read_sensor();
  if (reading.valid) {
    Serial.println("PASS");
    Serial.printf("  Voltage: %.2fV\n", reading.voltage);
    Serial.printf("  Current: %.2fA\n", reading.current);
    Serial.printf("  Power: %.2fW\n", reading.power);
  } else {
    Serial.println("FAIL");
  }
  
  // Test 4: MQTT publish
  Serial.print("Test MQTT Publish: ");
  bool published = publish_reading(reading.voltage, reading.current, 
                                   reading.power, reading.energy);
  if (published) {
    Serial.println("PASS");
  } else {
    Serial.println("FAIL");
  }
  
  // Test 5: Mode change
  Serial.print("Test Mode Change: ");
  change_operating_mode("realtime");
  if (current_mode == MODE_REALTIME) {
    Serial.println("PASS");
  } else {
    Serial.println("FAIL");
  }
  
  Serial.println("=== TESTS COMPLETE ===\n");
}

### 3.3 Load Tests (Multiple Devices)

Test Setup: - 10+ devices simultaneously - All in realtime mode (2 sec interval) - Monitor MQTT broker throughput - Check for message loss

Expected Results: - Message delivery rate > 99% - Average latency < 500ms - No device crashes after 24h - Memory usage stable

### 3.4 Power Consumption Tests

Test Procedure:

1. Connect multimeter in series with VCC
2. Measure current in each mode
3. Calculate daily energy consumption
4. Verify against specifications

Expected:
- Off mode: ~5 mA
- Lowpower: ~15 mA avg
- Interval: ~30 mA avg
- Realtime: ~80 mA avg

### 3.5 Security Tests

Checklist: - [ ] TLS 1.2 encryption verified - [ ] Certificate validation working - [ ] No hardcoded secrets (use EEPROM) - [ ] Reset protection with token - [ ] Physical button requires 10s press - [ ] MQTT authentication required - [ ] No plaintext WiFi passwords in logs

## 4. Deployment Guide

### 4.1 Initial Flashing

Prerequisites: - Arduino IDE or PlatformIO - USB cable (micro-USB or USB-C) - Driver for CH340/CP2102 (if needed)

Steps:

# 1. Install ESP8266 board support
# Arduino IDE: File → Preferences → Additional Boards Manager URLs
# Add: http://arduino.esp8266.com/stable/package_esp8266com_index.json

# 2. Select board
# Tools → Board → ESP8266 Boards → NodeMCU 1.0 (ESP-12E Module)

# 3. Configure settings
# Tools → Flash Size → 4MB (FS:2MB OTA:~1019KB)
# Tools → CPU Frequency → 80 MHz
# Tools → Upload Speed → 115200

# 4. Compile
# Sketch → Verify/Compile

# 5. Flash
# Connect device, select port, click Upload

Alternative (Command Line):

arduino-cli compile --fqbn esp8266:esp8266:nodemcuv2 firmware.ino
arduino-cli upload -p /dev/ttyUSB0 --fqbn esp8266:esp8266:nodemcuv2 firmware.ino

### 4.2 Configuration (First Boot)

Via Serial:

void setup() {
  Serial.begin(115200);
  
  if (!config_loaded()) {
    Serial.println("\n=== FIRST BOOT CONFIGURATION ===");
    Serial.println("Enter WiFi SSID:");
    while (!Serial.available());
    String ssid = Serial.readStringUntil('\n');
    
    Serial.println("Enter WiFi Password:");
    while (!Serial.available());
    String password = Serial.readStringUntil('\n');
    
    Serial.println("Enter MQTT Broker:");
    while (!Serial.available());
    String broker = Serial.readStringUntil('\n');
    
    save_config(ssid, password, broker);
    Serial.println("Configuration saved!");
  }
}

Via AP Mode (Web Interface):

- Device creates WiFi AP: “ControlaLuz-Setup”

- Connect and open http://192.168.4.1

- Enter WiFi and MQTT credentials

- Device saves and reboots

### 4.3 OTA Updates (Production)

Process:

1. Admin uploads firmware to S3 (backend)

2. Backend sends MQTT command to device(s)

3. Device downloads and flashes firmware

4. Device reboots with new version

5. Device reports success/failure

Advantages:

- No physical access required

- Update multiple devices simultaneously

- Automatic rollback on failure

- Version tracking

## 5. Troubleshooting Guide

### 5.1 Common Issues

| Issue | Cause | Solution |
| --- | --- | --- |
| Won’t connect to WiFi | Wrong password | Check credentials in EEPROM |
| MQTT connection fails | Certificate issue | Verify certificates, check IAM policy |
| Sensor returns NaN | PZEM disconnected | Check UART wiring, power |
| Random reboots | Power supply | Use stable 5V 1A+ supply |
| OTA fails | Low memory | Close other connections first |
| Won’t boot after OTA | Corrupt firmware | Bootloader auto-rollback |

### 5.2 Debug Serial Output

// Enable verbose logging
#define DEBUG_ESP_PORT Serial
#define DEBUG_ESP_WIFI
#define DEBUG_ESP_TLS
#define DEBUG_ESP_HTTP_UPDATE

// In code
Serial.setDebugOutput(true);

## 6. Complete Testing Checklist

### 6.1 Feature Tests

- WiFi connection (WPA2)

- MQTT connection (TLS)

- MQTT publish readings

- MQTT receive commands

- Sensor reading (all parameters)

- Operating mode change (all modes)

- OTA update (success case)

- OTA rollback (failure case)

- Reset protection (valid token)

- Reset protection (invalid token)

- Physical button reset

- LED indicators (all states)

- EEPROM persistence

- Auto-reconnect (WiFi)

- Auto-reconnect (MQTT)

### 6.2 Performance Tests

- Memory usage < 60 KB

- Flash usage < 500 KB

- Reading interval accuracy

- MQTT latency < 500ms

- WiFi signal strength monitoring

- 24-hour stability test

- Power consumption verification

### 6.3 Security Tests

- TLS 1.2 encryption

- Certificate validation

- Reset token validation

- Physical button 10s delay

- No secrets in logs

- MQTT authentication